Pre-processing:

Japanese OBS data,

Møre 2009 Survey

UiB Bergen

November, 6th 2009

Trond Kvarven

Department of Earth Science, University of Bergen

Allegaten 41, N-5007 Bergen, Norway

Introduction	3

Experiment setup	4

Receiver type	5

Data processing	5

Data loss	5

Data quality	6

Channel assignment	6

Display of seismic sections	6

Acknowledgement	6

References	7

Appendix 1. Tables	8

Appendix 2. Seismograms	10

## Introduction

The acquisition of the three crustal scale OBS profiles (figure 1) was performed during a cruise with M/V “Håkon Mosby” from May 24th to June 9th, 2009. Several institutions participated: UiB, UiO, Hokkaido University and Leibniz Institute of Marine Sciences at the Christian-Albrechts University of Kiel (IFM-GEOMAR). The data were collected as a part of a Collaborative Research Project (CPR) within the multi-national and multi-disiplinary TopoScandiaDeep project. The CPR is called "The Scandinavian mountains: deep processes" and is aiming to understanding the ongoing dynamics of the Møre region, mid-Norway.  In addition several land-stations were deployed in the landward continuation of profile 1 and 2. The survey was accompanied by gravity and magnetic measurements. This report describes the pre-processing of the data collected by the Hokkaido OBSs. Consult the cruise report (Mjelde & Minakov, 2009) for more details about the data acquisition.

The main objectives of this CPR project are:

- Mapping of the Jan Mayen Lineament (JML)

- Mapping the possible onshore continuation of the lower crustal high velocity bodies (LCB).

- Investigate if bodies of Caledonian eclogites are present within the lower crust near the coastline

Investigate the interplay between the Møre-Trøndelag Fault Complex (MTFZ) and the inherited structures.

Figure 1. The study area of the OBS survey Møre Basin 2009. The shot profiles are shown by black solid lines. Purple triangles point to the OBS deployment position. From Mjelde & Minakov (2009).

Figure 1. The study area of the OBS survey Møre Basin 2009. The shot profiles are

shown by black solid lines. Purple triangles point to the OBS deployment position. From Mjelde & Minakov (2009)

## Experiment setup

The seismic source comprised four equal seized 1200 inch3 air-guns with a total volume of 4800 inch3. The shooting interval was approximately 200 meters.

For the 281 km profile 1, the OBSs no. 101-105 were provided by the Hokkaido University, while OBSs no. 106-116 were provided by Geomar. For the 293.2 km profile 2, the OBSs no. 201-205 were provided by the Hokkaido University, while OBSs no. 206-216 were provided by Geomar. For the 206.2 km profile 3, the OBSs no. 304, 309, 311, 312 and 313 were provided by the Hokkaido University, while OBSs no. 303, 305, 306, 307, 308 and 310 were provided by Geomar. The OBSs provided by the Hokkaido University were used in the most landward position and were also deployed in a denser interval than the Geomar OBSs (figure 1).

## Receiver type

The OBSs from Hokkaido University consists of 3 geophones, while all of the GEOMAR OBSs were 4 component systems with one hydrophone in addition to the 3 geophones.

## Data processing

Before the pre-processing two text files were prepared: Cutout.txt and Shot.txt for input when cutting the continous recording into traces with correct length (usually 60 seconds for regional OBS surveys. The main body of the processing is codat. This program reads stored raw-files and performs resampling, synchronising and cutout of shot windows (60 seconds windows). Output sampling frequency is 256 Hz. The program obshd2ut was used to correct

the.inf files for drift in the OBS clocks. Exact shot timing is combined with the seismic data

during conversion to SEG-Y format (programs shot2info_dst and obsconv). The program relobs.hyb.csh is used for relocation of the OBSs. The water depth were, however, to shallow for a successful relocation (max 289 metres water depth).

Data loss

Figure 2. Work flow of the pre-processing of the data collected from the Geomar OBSs.

All 43 OBSs deployed were recovered (actual 41 since OBS 104 and 204 are identical with OBS 309 and 304 respectively). 3 OBSs had problems: OBS 201, 203 and 205 did contain reduced amount of useful data (table 1). Due to problems with the compressor and poor weather conditions, 19.8 km of profile 1 was acquired with 2 air-guns only, and the rest of the profile (261.2 km) was acquired with 3 air-guns. A total of 36 shots were lost and 76.6 km were acquired with low pressure (120-125 bars), due to compressor problems. Profile 3 was acquired without incident, but two OBSs towards each end of the profile were not deployed, due to the time loss. The above implies the following: 1% of the data along the profiles was lost, 2.5% of the profiles were acquired with 2 air-guns, 33% with 3 air-guns, and 10% was acquired with low pressure. Consult the cruise report (Mjelde & Minakov, 2009) for more details about the data acquisition.

## Data quality

The quality of the processed data (airgundata only) was analysed by visual inspection of the

seismograms shown in Appendix 2. The signal range and the amount of noise in the data is

what decide the quality. Good quality data have signal range greater than 80 km while data

with signal range less than 40 km is considered poor/bad.

The overall quality of the data can be regarded as moderate to very good (Table 3, Appendix 1).

### Channel assignment

Channel 1 = Vertical low gain

Channel 2 = Horizontal 1

Channel 3 = Horizontal 2

Channel 4 = Vertical high gain

### Display of seismic sections

The seismic stations are displayed in standard mode following:

8 km/s reduced traveltime

Bandpassfilter 2/5/15/20 Hz

Automatic gain control with 4 seconds AGC window

Offset range -200 to 200 km

Vertical component time window = 0-12 seconds

Horizontal component time window = 0-12 seconds

## Acknowledgement

The data acquisition was funded by the Research Council of Norway (NFR). We would like to thank all the participating institutions and the crew at R/V Håkon Mosby. Thanks to Yoshio Murai at ISV Hokkaido University and Audun Libak at the University of Bergen for all help during the preprocessing.

## References

Mjelde, R. & Minakov, A. 2009. Cruise Report: Møre 2009: OBS survey Møre Basin within the Toposcandia project by use of Håkon Mosby.  Department of Earth Science, University of

Bergen, Norway.

## Appendix 1. Tables

Table 1. Shot statistics after processing

| Station # | Device name | Gaps | # of shots achieved | # of shots lost | # of shots performed |
| --- | --- | --- | --- | --- | --- |
| 101 | dr | 541, 902, 916-922 | 1396 | 9 | 1405 |
| 102 | di | 541 | 1404 | 1 | 1405 |
| 103 | dj | 263-268, 541 | 1398 | 6 | 1405 |
| 104 | dxd | 541 | 1404 | 1 | 1405 |
| 105 | dc | 541 | 1404 | 1 | 1405 |
| 201 | dk | Several | 729 | 752 | 1481 |
| 202 | dh | Complete | 1481 | - | 1481 |
| 203 | da | Several | 570 | 911 | 1481 |
| 204 | dxg | Complete | 1481 | - | 1481 |
| 205 | dxa | 2715-2954 | 1241 | 240 | 1481 |
| 304 | dxg | Complete | 1032 | - | 1032 |
| 309 | dxd | Complete | 1032 | - | 1032 |
| 311 | dxe | Complete | 1032 | - | 1032 |
| 312 | dxb | Complete | 1032 | - | 1032 |
| 313 | de | Complete | 1032 | - | 1032 |

Table 2. Clock drift and synchronization times

| Station # | Device name | Start date | Start time | End date | End time | Drift (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| 101 | dr | 28.05.2009 | 3h42m | 31.05.2009 | 00h05m | 207 |
| 102 | di | 28.05.2009 | 3h15m | 30.05.2009 | 22h45m | -19 |
| 103 | dj | 28.05.2009 | 2h29m | 30.05.2009 | 21h393m | 18 |
| 104 | dxd | 28.05.2009 | 1h49m | 07.06.2009 | 07h35m | 376 |
| 105 | dc | 28.05.2009 | 0h50m | 30.05.2009 | 19h39m | 158 |
| 201 | dk | 02.06.2009 | 05h24m | 05.06.2009 | 02h05m | -52 |
| 202 | dh | 02.06.2009 | 04h49m | 05.06.2009 | 00h34m | 409 |
| 203 | da | 02.06.2009 | 04h06m | 04.06.2009 | 23h38m | 74 |
| 204 | dxg | 02.06.2009 | 03h05m | 07.06.2009 | 00h03m | 179 |
| 205 | dxa | 02.06.2009 | 02h08m | 04.06.2009 | 21h17m | 82 |
| 304 | dxg | 02.06.2009 | 03h05m | 07.06.2009 | 00h03m | 179 |
| 309 | dxd | 28.05.2009 | 1h49m | 07.06.2009 | 07h35m | 376 |
| 311 | dxe | 05.06.2009 | 15h30m | 07.06.2009 | 09h52m | 85 |
| 312 | dxb | 05.06.2009 | 16h36m | 07.06.2009 | 11h13m | 225 |
| 313 | de | 05.06.2009 | 17h36m | 07.06.2009 | 12h34m | -7 |

Table 3. Data quality

| Station # | Vertical low gain data quality | Horizontal 1 data quality | Horizontal 2 data quality | Vertical high gain data quality |
| --- | --- | --- | --- | --- |
| 101 | 0 | 0 | 0 | 0 |
| 102 | ++ | 0 | 0 | ++ |
| 103 | - | - | - | - |
| 104 | -- | -- | -- | -- |
| 105 | - | - | - | - |
| 201 | Noise | Noise | Noise | Noise |
| 202 | 0 | 0 | 0 | 0 |
| 203 | Noise | Noise | Noise | Noise |
| 204 | 0 | 0 | -- | 0 |
| 205 | + | + | -- | + |
| 304 | + | 0 | 0 | + |
| 309 | ++ | ++ | ++ | ++ |
| 311 | ++ | ++ | ++ | ++ |
| 312 | ++ | ++ | ++ | ++ |
| 313 | ++ | ++ | ++ | ++ |

| ++ | Very good (more than 120km signal range, clear arrivals, few noise) |
| --- | --- |
| + | Good (80 to 120 km signal range, more noise) |
| 0 | Moderate (< 80 km signal range, partly very noisy) |
| - | Poor (< 40 km signal range, noise dominates) |
| -- | Bad (< 20 km signal range, noise dominates) |
| Noise | Noise only |

## Appendix 2. Seismograms