Wireless Network Workshop / Exercise / Demonstration

Purpose: This workshop will be in several parts.  In the first part we will install radio cards in computers and assign IP's.  We will configure AP's and clients,  Then we will make a variety of different network configurations between computers, AP's and backbones.   We make some calculations of  “system operating margin”, gain and EIRP

Description:   In the first part of this exercise we will calculate the SOM, gain in DB, and EIRP for a 20 km link with and without amplifiers and different antennas.  In the second part we will install wireless LAN cards in laptops configure them and assign IP's and make a peer to peer link between two laptops.  In the third part two different groups will make a network of AP's and clients.  In the fourth part of the we will link two small networks with a backbone.

Equipment to need will be laptops, 802.11 cards, USB clients, AP's, Ethernet converters, various antennas and cables.  In your on the campus FTP there are a variety of spec sheets for this equipment along with an article on Microwave links. You also have several programs to help in the calculations we will be doing.

First part: Each student will given a random problem with a certain equipment and distance configuration and will be asked to say if the link will work and if not make a suggestion of what to do.  Formulas at the end of this document. (Time alloted for this part 30 minute).

Calculation needed.

Free space loss

system operating margin (SOM)

Fresnel zone

Effective Isotropic Radiated Power (EIRP)

Second Part:  In this section we will give students wireless 802.11b LAN cards and they will install then in laptops.  Assign IP's and configure a peer-to-peer network with the laptops in their group and perform a throughput test using Qcheck. (30 minutes max)

(Details here)

In the third part the particpants will change their computers to Infrastructure mode and connect to an AP and then do some Qchecks between machines in their group.  (30 minutes)

(Details here)

In the last part the participants will connect the two local networks to each other and make throughput test between the two networks. (30 minutes)

(Details here)