Chirinet seismic network

# Angel Rodriguez

Volcn, Chiriqui

Panama

angel@tierrasaltas.net

Chirinet is a privately owned and operated research seismic network.  It presently consists of 3 to 5 short period, three  component stations generally surrounding Volcan Baru.  It has one broadband sensor and one micro aperture, eight station array. The stations are all connected by an ethernet  wide area wireless network and connected to a central station at BRU2, that gathers data from remote stations and makes that data available to anyone via FTP. The station particulars are:

BRU2 is a GeoDevice broadband sensor with 24 bit Seisad18 digitizer

AVB0 it the central 3c station of the array, 16 bit SARA digitizer

BAG2 is a 3c 4.5 Hz station, 16 bit SARA digitizer

VEB2 is a 3c 4.5 Hz station, 16 bit SARA digitizer

TBS2 is a 3c 4.5 Hz station, 16 bit SARA digitizer

Radio link 1-5 is a YDI 5.8 gHz backbone

All other links are 2.4 gHz

Radios 6 and 10 are Karlnet AP's

The station at site 7, (BRU2) is where the central data collection takes place using  MiniSeisNet (MSNet). MSNet polls all stations every half hour.  It also has a public FTP server at chirinet.serverftp.org where the events files are available to anyone.   The network detects about 8 events per day