*  Choose a computer that has a “restart on return of power feature”

*  If at all possible choose a computer that has no fan

*  there is no need for a CD or floppy

*  500 mHz

*  At least one serial port

*  At one USB port

Set bios options as follow:

At the “Standard CMOS features”

make “halt on” = NO ERRORS

At the “Advanced BIOS fearures”

make “virus warning” = disable

make “First boot device” = HD0

make “”screen logo” = disable

At the “Power Management”

goto “Power manament”

make Power management = User defined

make HDD power down = 3 min

make DOSE mode = disable

make Sleep = disable

make Standby = disable

make “AC loss auto restart” = Enabled

make “Video Off Option” = Disable

Install Windows 2000 with NTFS file system

SP4 or most current SP and ALL updates

Set the system for auto logon

Disable certain services (see apendix)

Set Serial Port Enumerations

Enable the telnet service make a user to logon

On the desktop do right click:

then Properties

then Screen saver

then click on the Power button

then the Power Schemes tab

then set “turn off monitor” = Never

then set “turn off hard disk” = Never

The set “System stand by” = Never

Save

Install and configure the following software

TightVNC

K9 / Tardis

If you have a GPS and not interfaced to Seislog then install TAC32

Tlist and Kill in the root direcotry

psshutdown.exe

ActiveState Perl

nnCronLITE

WarFTP sever

WinRAR or WinZIP (I like WinRAR)

Qcheck “end point” (optional)

Seismowin

seisctl.exe