| GS-11D Specifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Natural Frequency | 4.5 ± .75 Hz | 8 ± .75 Hz | 10 ± .75 Hz | 14 ± .75 Hz |
| Coil Resistance @ 25°C ± 5% | ——380 Ohms —— |  |  |  |
| Intrinsic Voltage Sensitivity with 380 Ohm Coil ± 10% | ——.81 V/in/sec (.32 V/cm/sec) —— |  |  |  |
| Normalized Transduction Constant (V/in/sec) | ——042 (sq.root of Rc) —— |  |  |  |
| Open Circuit Damping | .34 ± 20% | .39 ± 10% | .32 ± 10% | .23 ± 10% |
| Damping Constant with 380 Ohm Coil | 762 | 602 | 482 | 344 |
| Optional Coil Resistances ± 5% | ——56,160 Ohms ——— |  |  |  |
| Moving Mass ± 5% | 23.6 g | 16.8 g | 16.8 g | 16.8 g |
| Typical Case to Coil Motion P-P | .07 in | .07 in | .07 in | .07 in |
|  | (.18 cm) | (.18 cm) | (.18 cm) | (.18 cm) |
| Harmonic Distortion with Driving Velocity of 0.7 in/sec (1.8 cm/sec) P-P | N/S | ————0.2% or less ———— |  |  |
|  |  | @ 12 Hz | @ 12 Hz | @ 12 Hz |
| Dimensions |  |  |  |  |
| Height (less terminals*) | —————1.32 in (3.35 cm) ————— |  |  |  |
| Diameter | —————1.25 in (3.18 cm) ————— |  |  |  |
| Weight | —————3.9 oz (111 g) ————— |  |  |  |
| *terminal height is .135 inches |  |  |  |  |
| Click HERE to load chart for Seismic Detector Response Curve Output VS. Frequency (GS-11D @ 4.5 Hz @ 380 Ohms & GS-11D @4.5 Hz @ 4000 Ohms). (115 KBytes) |  |  |  |  |
| Click HERE to load chart for Seismic Detector Response Curve Output VS. Frequency (GS-11D @ 10 Hz @ 380 Ohms). (60 KBytes) |  |  |  |  |