UNIVERSITETET I BERGEN

Matematisk-naturvitenskapelig embetseksamen

Subject: GFJ 372 Seismic Instrumentation

Friday, April 14, 2000,  0900-1400 hrs.

Permitted remedy: Calculator, English dictionary

A total of 100 points is possible for this exam. The number of points for each question is indicated.

# Question 1, 10 points

Set up an experiment to measure the internal resistance of a battery using a multimeter and an external resistor.

- If two batteries of the same type have different internal resistance, what does this tell you about the state of charge.

- A car battery has an unloaded voltage of 12.44 V. What is the approximate % charge.

Question 2, 5 points

3 resistors are connected to a 12 V battery as shown below.

------------------

|

|

|

12 V            -------|-------

|      |      |

R1     R2     R3

|      |      |

-------------------------

R1= 3 ohm, R2= 4 ohm, R3= 6 ohm

- Calculate the current through each resistor.

- What is the equivalent resistance to the 3 resistors in parallel.

Question 3, 8 points

A solar panel gives out 20 V. When shorted, the current is 1 A. When connected to a car battery, the voltage is 14 V.

- Calculate solar panel internal resistance.

- What is the charging current ?

Question 4, 12 points

A weight of 10 g is lifted off a seismometer. This gives a pulse of 20 mV amplitude out of the signal coil. A DC pulse of 20 mA into the calibration coil gives an amplitude of 30 mV.

- Calculate the seismometer calibration coil motor constant. If the seismometer mass is 2 kg, the generator constant is 200 V/(m/s) and a sine wave of 10 mA (p-p) at 10 Hz is input to the calibration coil, what is the equivalent earth displacement (p-p) at 10 Hz.

Question 5, 8 points

A seismometer has a natural period of 5 secs, a generator constant of 200 V/(m/s). It is connected to an oscilloscope set to a sensitivity of 1mV pr division.

- How large must the earth velocity (p-p) at 10 Hz be in order to be able to see the signal on the oscilloscope with an amplitude of 6 divisions (p-p).

Question 6, 3 points

A digital voltmeter has a range from -20000 mV to 20000 Mv.

- How many bits does the A/D converter in the voltmeter have.

Question 7, 10 points

- Give a general description of a triggered data acquisition system like SEISLOG or GBV316.

Question 8, 8 points

- Explain why a 4.5 Hz geophone gives very similar measurements as a 1.0 Hz seismometer considering the earth noise.

Question 9,  8 points

- Construct a 2 pole RC band pass filter with frequencies 5 and 10 Hz.

Question 10, 3 points

A seismometer has a critical damping resistance of 5 kohm and an internal resistance of 3 kohm.

- If the desired damping is 0.6, calculate the damping resister to use.

Question 11, 10 points

- Give an example of a one octave filter.

- What is the relation between acceleration power

spectral density and displacement power spectral density ?

- If displacement power is proportional to d*d/f0 (2/3 octave filter) where d is rms displacement amplitude and f0 is filter center frequency, what is the relation between    d and acceleration power spectral density ?

- If the RMS displacement is 1 nm at 1 Hz measured with a 2/3 octave filter, what is the acceleration power spectral density ?

- Convert the acceleration power spectral density to dB relative to 1 (m/s**2)**2)/Hz.

Question 12, 2 points

- An amplifier has a gain of 66 dB, how many times is the amplification ?

Question 13, 3 points

A 1g accelerometer with a max output of 10 V is inclined 10 degrees from vertical.

- What is the output from the vertical channel ?

Question 14, 3 points

A digital seismic station has 20 channels of data. The sample rate is 200 samples/sec and a 16 bit A/D converter is used. The data is compressed by a factor of 2. The recording is continuous.

- How much data (Mbytes) are recorded in 24 hours ?

Question 15, 2 points

- When increasing the damping of a seismometer, how is the free period changing ?

Question 16, 2 points

- What is typical signal and calibration coil resistances respectively ?

Question 17, 3 points

- What is a typical bandwidth and dynamic range of an accelerometer.