##### Data available at the ##### Norwegian National Earthquake Data Base ##### at Department of Earth Science by Seismo Group Department of Earth Science University of Bergen Allégaten 41 N-5007 Bergen e-mail: seismo@geo.uib.no Phone: (+47) 55 58 36 00 Fax: (+47) 55 58 36 60 May 2011

# Introduction

Earthquake data has been collected at the institute since late eighteen hundred. Initially the data was only felt reports, but starting with the first seismic station in Bergen in 1905, also instrumental data has been collected. The information of felt events is still being collected and most of the original data is available. Instrumental data is being collected from the Norwegian National Seismic Network (NNSN) and NORSAR as well as some data from neighboring countries and international data banks. The instrumental data consist of both parameter data (the catalog and bulletin) and digital  waveform data. Our goal is that the Norwegian National Earthquake Archive (NNEA) shall continue to contain the most complete information available about seismic events in our area as well as having archives for the physical material (seismograms, bulletins etc). Currently, ALL data available in electronic form is also available on our web page and ftp server.

NNEA currently (April 2003) contains information about 75 000 seismic events.

NNEA is constantly getting data added and periodically revisions  is being made of available data to improve the quality. The purpose of this report is to document the overall content of the data base and changes that has taken place as well as known problems to be solved. The report will be updated periodically.

Changes:

May 2011 alk: Missing ISO files have been regenerated

# Data

NNEDB contains different types of material.

Felt earthquakes: All reports on felt earthquakes (including macroseismic maps) are listed in different yearly reports and special reports. After 1985, this information was included in the printed bulletin. The felt earthquakes are also listed in the catalog, see below. This material is  available in printed form and partly in electronic form. The original questionnaires for most of these events are available in our archive. See below for an overview of printed material available as well data available electronically (section Bulletins and macroseismic data)

Bulletins: Printed reports of seismic events observations from seismograms. Since the first station was installed, bulletins have been printed and widely distributed. See below for an overview of available bulletins.

Data base:  The electronic data base (DB) of parameter information about seismic events has been assembled from various sources over the years and contain references to most events, that we know about in the area.

Waveform analog and digital data: All original seismograms are available in our archives (a few single seismograms are missing, no overview of that available), starting with data from 1905. Digital data is available from 1982. No listing will be given of this material since available data  is cross referenced in the DB.

A more detailed description will now be given of the available data sources.

# The Data Base

The data base originally started with whatever data was read on any of the stations operated by NNSN for both local and global earthquakes. The amount of data for southern Norway increased when the Western Norway network was installed in 1984 and similarly it increased in N. Norway when the Northern Norway network was installed in 1988. Improved station coverage on Jan Mayen and Svalbard also increased the detectability in the Norwegian Sea. The NORSAR arrays has in recent years contributed a significant amount of data not seen by NNSN, particularly in the eastern areas. This means that the area covered  has increased over the years and the DB is non uniform in areal coverage as well as time coverage. This has led to a definition of a prime area for the DB:

Latitude 	  54.0	82.0

Longitude	-15.0	36.0

Within that area, it is the aim to have the most complete data base possible. Teleseimic events in the data base will only include events recorded on Norwegian stations and there is no attempt to have any kind of completeness in either time or space. Teleseismic data from before 1982 is not included and NORSAR teleseismic data is only included from January1997. Before 1982, only a few phase readings from Norwegian stations are included. These reading will, however be found in the printed bulletin (see bulletin section).

# Data base organization

All data are in the main data base called BER since BER is the official agency name for NNSN. From 1900 and onwards, there are individual S-files for each event. Before 1900, there is only one CAT file. From 1964, the data base also have phase readings, mostly from ISC, however no relocation has been made before 1980 so the data base has been updated with UPD (meaning that no relocation has been made).

# Data base components

Before 1964: The prime catalog is from NORSAR (NORSAR and NGI, 1998). The catalog only contain hypocenters. In addition, the data from Singh et al (1990) for Southern Norway has been integrated retaining the original determination. The Singh catalog added 5 new events  in the time period 1900-1963. Several duplicates were removed from the NORSAR catalog.

1964-1969: The prime data is the NORSAR hypocenter. In addition, all available ISC data  with phase readings have been added, but no relocations made. The first line in the file will contain the NORSAR hypocenter. The Singh catalog has also been integrated and one new event was added from this catalog (196502100518). Table 1 shows the statistics of the merging of the 3 catalogs up to 1969.

Table 1  Statistics of merging NORSAR, Singh and ISC catalogs. The columns show the number of events in the original catalogs and the number after merging (total).

Year(s)       ISC NORSAR Singh  Total

before 1900         1039    44   1039

1900-1963           1340   158   1336

1964   173     48     4    177

1965   178     43     4    192

1966   186     62     3    191

1967   211     70     1    220

1968   220     74     2    240

1969   257     84     1    275

1970   329     83     1    339

1971   161     56     3    171

1972    71     47     2     83

1973   220     54     0    226

1974   231     77     6    232

1975   122     59     3    128

1976   122     65     2    127

1977    77     57     3     87

1978   168     87     2    175

1979   109     64     1    115

The ISC catalog primarily adds data in the arctic areas and smaller events reported by Uppsala or Helsinki

1980-

From 1980 to 1984, the original catalog primarily consisted of readings from the  BER stations and data from the Southern Norway network. This was clearly very incomplete. It is not clear if all data from the Southern Norway network was included, however it does not seem to be included in the ISC bulletin.  The aim is now that the data base should contain at least all events (with readings) with magnitude larger than 3.0 in the prime area from 1980 and onwards. It was therefore not possible to use the NORSAR catalog which only contains prime hypocenters.  The ISC was therefore used and all events with magnitude larger than 3 in the ISC catalog was selected for merging with the BER catalog. The aim was also to be able to relocate the BER data base from 1980 and onwards.

# Clean up for the periode 1980-99

NORSAR amplitudes: Before starting the merging, some cleanup was performed (February 2000). All NORSAR amplitudes associated with  P-Phases were removed since it is impossible to recalculate magnitudes with these amplitudes with recognized magnitude scales. The amplitudes resulted in very large magnitude estimates.

Wrong phase lines: Removed

Waveform file names: All old Vax names were corrected to the current Unix names and all waveform files can now be plotted directly. All waveform files are online on the WAV waveform directory.

Status of merging ISC data: Currently, the data has been included until and including June 1982.

Principle for merging:

- Delete duplicate readings from ISC

- Keep ISC header as a second hypocenter line

- If only located by ISC, duplicate ISC header so original ISC location is available after relocation (ISC do not locate with S for local earthquakes)

- After merge, locate and check. If location difficult, lock to best of original locations

-     Put  magnitude to 1. line, 3 . position if no magnitude calculated or missing

The KOLA data base

NORSAR has worked on a project on seismicity on KOLA (Bungum and Lindholm, 1997). The data has been integrated into the BER data base and the original data is available in the KOLA data base. The Kola database extends from 1979 to 1992.

1. For 1979, only headers from the KOLA data are included in the BER data, under the agency name KOL

2. For 1980-1991 all duplicate events are compared. For events that have identical readings available for both KOLA and BER, only KOLA (Agency KOL) are included in the BER S-files.

3.Readings that are available in the Kola data, but not for the same events in BER, both additional readings and headers (Agency KOL) are appended to the event in the BER database. Thereafter the event has been relocated.

4. For new events, the KOLA data are simply included in the BER database (these events have not been manually relocated). The header line is repeated twice so that the original Kola location will not be lost during future updates.

The 1992 events have yet to be included.

During relocation, station name MYV has been changed to MYR. There do not exist station parameters for a station MYV (MYV=MYR). The statistics of the merge is seen in Table 2.

Table 2, Statistics of merging KOLA and BER data bases. First column (KOLA) gives number of event in KOLA data base, BER ok gives identical events in BER data base, Merge is merging was done and New are completely new events.

KOLA     BER ok	Merge	New

1979	  12  			 1

1980	  13	     11	  1	 1

1981	   9	      8	  1	 0

1982	  14	     12	  2	 0

1983	  26	     22	  2	 2

1984	  50	     38	 12	 0

1985	  25	      	  8	17

1986	  18	      	  4	14

1987	  34	      1	 13	20

1988	  39	      4	 15 	20

1989	  66	      3	 20	43

1990	  46	      5	 13	28

1991	  34	      6	 11	15

1992   132

# Explosions

The majority of the events in the data base originating at the Norwegian mainland, are explosions. The institute receives regular reports of explosion and these are marked with E in the first header line (see Nordic format description). In addition, the file contains information about the size of the explosion and the time and location of the explosion, if reported. Many explosion occur at almost the same location at regular intervals, and these have been assigned a code (see below). The majority of explosion are not reported. However, many sites with regular explosion activity (same time of the day) have signals easily recognized by the operator and are marked as probably explosion in the data base (P instead of E). A few of these might be earthquakes. Currently (April 2004) there are 1800 confirmed explosions in the data base and 17000 events marked as probable explosions.

Some explosion sites:

| Name | Code | Latitude | Longitude |
| --- | --- | --- | --- |
| Haakonsvern | HAA |  |  |
| Titania | TIT | 58.3 | 6.4 |
| Blåsjø | BLS | 59.2 | 6.9 |
| Rogaland Sjøforsvarsdistrikt | RSD |  |  |
| Minedykkergruppen, Sør-Norge | MDT |  |  |
| Forsvarskommando Sør Norge | FKS |  |  |
| Rana Gruber, Mo i Rana | RAN |  |  |

For other explosion reports, the code used is OTH. The information about explosion sites was first entered into the data base in the fall of 2003. During this process, all events marked with E, and with no confirmed explosion record, had the E removed. Most events from the Kiruna/Malmberget area were manually given the P-flag. Events with only Norsar data and the E flag, were changed to P.

# Bulletins and macroseismic data

Reports of felt earthquakes was started in 1888 in series called ‘ Jorskjelv i Norge’ which continued until 1952. From 1953 to 1984, special reports were issued and from 1985, the macroseimic data was included with the bulletin. These felt reports also initially contained the bulletins but later individual station bulletins were made. This reporting was replaced by weekly bulletins of all stations in 1967 and finally by monthly bulletins in 1982. The details are seen below.

| 1834-1892 | Handwritten notes |
| --- | --- |
| 1888-1952 | Jordskjelv I Norge, year 1894 is missing |
| 1953-1975 | A catalogue of earthquakes felt in Norway 1953 – 1975 and A statistical analysis related to the completeness of catalogues for earthquakes felt in Norway 1899 – 1979 by Markvard A. Sellevoll, Jan Almaas and Andrzej Kijko |
| 1975-1983 | Seismo-Series No. 3 A catalogue of earthquakes felt in Norway 1975 – 1983 and a comparison of instrumental and macroseismic studies and epicenters by Jens Havskov and Per M. Optun |
| 1984 | Seismo-Series No. 17 A catalogue of earthquakes felt in Norway 1984 by Per M. Optun |

Table 3  Felt information. The original  material for the macroseismoic studies is avialble for the years 1758, 1819, 1866, 1879 - 1890  ( 1891- 1897 is missing ) and  from 1898 to present.

|  | Bergen | Isfjord | Tromsø TRO | Kongsberg KON | Kirkenes KRK | Jan Mayen JMI | Lillehammer LHN | KingsBay KBS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1905 | I Jordskjelv Norge |  |  |  |  |  |  |  |
| 1914 | Stop 31-12 |  |  |  |  |  |  |  |
| 1922 | Bergen bulletin | Start 1-8 |  |  |  |  |  |  |
| 1960 | ----- | Ok | Start 17-9 |  |  |  |  |  |
| 1962 | ----- | Stop 25-9 | Ok | Start 1-9 |  |  |  |  |
| 1964 | ------ |  | ok | Ok | Start 1-1 | Start 1-1 |  |  |
| 1965 | ------- |  | ok | Ok | Ok | Ok | Start 1-4 |  |
| 1967 | Stop 31-12 |  | Stop 10-10 | Stop 31-12 | Ok | Stop 31-12 | ok |  |
| 1967 | Preliminary weekly bulletin, start 1-9, PWB |  |  | Included in PWB | Ok + PWB from okt. |  | +PWB | PWB from sept. |
| 1968 | No 1,32,33,34,35,36,38 missing |  | Start in PWB 19-2 | PWB | Ok +PWB |  | +PBW | PWB |
| 1969 | ----------------- |  | PWB | PWB | Stop 10-5 + PWB |  | Stop 1-5 +PWB | PWB |
| 1970 | No 27 missing |  | PWB | PWB |  |  |  | PWB |
| 1971 | No 18,19 missing |  | PWB | PWB |  |  |  | PWB |
| 1972 | No 1 missing |  | PWB | PWB |  |  |  | PWB |
| 1982 | Stop 1-12 |  | PWB | PWB |  |  |  | PWB |
| 1982- | Start monthly bulletins with all available stations |  |  |  |  |  |  |  |

Table 4  Available printed seismic bulletins. For each station/network start and stop of bulletins are indicated

Scanned copies of all macroseismic maps are available in the Scandinavian Earthquake Archive is found at http://www.geo.uib.no/seismo/REPORTS/Scandinavian_earthquake_archive/. The archives cover the period from 1000 to 1985.

From 1975, the data used to produce the macroseismic maps are also available in digital form in terms of intensities at geographical locations. This felt earthquake data base was revised in 2007 and integrated with the normal data base making it possible to plot the intensity observations directly from SEISAN. In connection with this work, it was discovered that several observation files were missing, however the original material is available.

A complete overview of available data for the period 1975 to 2005 is given in Appendix 1. For this time period, there are reports of 262 felt earthquakes. Macroseismic maps were produced for 142 of these based of digital files. 44 files have been lost.

Checking RMS of the data base

It has been seen that several events are badly mislocated with high RMS of travel time residuals. In order to get an idea of the magnitude of the problem, some statistics of events with high RMS was made, see Table 3.

Table 3, RMS statistics. LR are local and regional events and D is distant events. The numbers are number of events larger than the RMS number given in the first line.

LR          D

>10	>20	>10	>20

1980	 2	 0	 1	 0

1981	 1	 1	 0	 0

1982	 8	 3	 2	 2

1983	 2	 1	 6	 3

1984	 4	 1	 2	 1

1985	 7	 1	 6	 5

1986	 5	 1	 4	 2

1987	10	 4	 7	 2

1988	 4	 1	 3	 1

1989	 1	 0	 2	 0

1990	41	17	 2	 1

1991	 3	 2	 3	 0

1992	 1	 1	 2	 0

1993	 0	 0	 0	 0

1994	 0	 0	 1	 1

1995	 0	 0	 1	 1

1996	 5	 0	 6	 1

1997	 6	 1	 2	 1

1998	 4	 1	 6	 3

1999 	 0	 0	 1	 0

Total  104	35	57	24

The table shows that 161 events have rms larger than 10. Obviously many events must be checked. Inspecting a few events with high rms, shows that the original location was ok, but change of location parameters or the location program is the probable cause of the large rms. Most of these event do not have bad readings, but the quantity and configuration of the stations, has made the location unstable.

The figures show statistics of the various data base as well as statistics of events with high rms.

# Clean-up of high RMS and inclusion of new NORSAR data in the database

A search was made in the BER database for events with rms higher than 3 in the time interval 01011990 to 31121999. 369 events were found. For the time interval 1993-1999, all events were inspected and corrected so that the rms was reduced.

In an attempt to clean up mislocated events with high RMS, a new file was sent from NORSAR including data from the period 01011990 to 31121999. The events in the NAO database were replaced by these new events.

Besides, all regional and local NORSAR events with a magnitude higher than 2.5 in the time interval 01011990 to 31121992 were split into the BER database. Duplets were deleted and NAO events were merged with BER events in cases where data were available from both companies. After this, there are still 253 events from 1990-1992 with rms higher than 3.

Review of parts of the Bergen database.

This review covers the time period from 1st January 1980 to 31st December 1999, and was made during the first 5 months in 2003. The review was made by Berit Marie Storheim (1980-1989) and Mathilde B. Sørensen (1990-1999).

All events in the Bergen database with rms > 3.0 and a location in the prime area ( 54-82 º N and 15º W–32 º E) were reprocessed. In total 761 (392 in 1980-1989 and 369 in 1990-1999) events were relocated. Comment lines explaining the cause of action are included in all events relocated in this review.

There was not any main cause for the large rms values, but the most common problems are listed below.

●	wrong start location

●	wrong station location ( NRA  at the end of 1989)

●	Change of input parameters for the location routine

●	Error when merging additional data (Kola)

●	The presence of several readings for the same phase at the same stations for a given event.

After the clean-up, the data base has been checked for high RMS values once again. The situation in May 2004 is listed in Table 4.

Table 4, RMS statistics. LR are local and regional events and D is distant events. The numbers are number of events larger than the RMS number given in the first line.

LR          D

>10	>20	>10	>20

1980	 0	 0	 0	 0

1981	 0	 0	 0	 0

1982	 0	 0	 2	 2

1983	 0	 0	 4	 1

1984	 0	 0	 2	 1

1985	 0	 0	 4	 3

1986	 0	 0	 4	 2

1987	 1	 0	 6	 1

1988	 0	 0	 2	 0

1989	 1	 0	 2	 0

1990	 0	 0	 2	 1

1991	 0	 0	 3	 0

1992	 0	 0	 2	 0

1993	 0	 0	 0	 0

1994	 0	 0	 1	 1

1995	 0	 0	 0	 0

1996	 0	 0	 5	 1

1997	 0	 0	 7	 5

1998	 0	 0	13	10

1999 	 0	 0	10	10

2000   0	 0	19	17

2001   0	 0	11	10

2002   0	 0	 6	 6

2003   0	 0	 3	 2

Total  2	 0	108	73

# Inclusion of KMS readings in the database

In connection with a study of earthquakes in Skagerrak, the complete database of readings from KMS, Denmark, was made available. These readings were merged into the BER database. Readings were available for 99 events in the time period 1995 0530 to 2003 1203. Each event was merged into the BER database manually by copying the Danish location and readings not already present in the BER database into the database. In addition to arrival times from the Danish stations, a large amount of Finnish readings were added. Events, which were not already present in the BER database, were copied into the database.

All events were relocated and checked.

Work to be done:

- Merge remaining ISC data from July 1982, M>=3.0

- Check against NORSAR catalog 1980-2000

- Check what is missing from Southern Norway Network

- Macroseismic data should be checked, format has changed and older data should be reformatted to the current Nordic standard

- Check Mb for all events in region, particularly when determined with local data, often wrong magnitudes

- Retype lost macroseismic data

# References

Bungum & Lindholm (1997): Seismo and neotectonics in Finnmark, Kola Peninsula and southern Barents Sea. Part 2: Seismological analysis and seismotectonics., Tectonophysics, 270, pp. 15-28.

NORSAR and NGI (1998): Seismic zonation of Norway. NORSAR Report 629.

Singh, S. K., M. Ordaz, C. D. Lindholm and J. Havskov (1990). Seismic Hazards in Southern Norway. Seismo-Series No 46, Institute of Solid Earth Physsics. University of Bergen.

Appendix 1

Felt earthquakes in Norway from 1975 to 2006

The first column gives the time (GMT). The second column indicates with ok if a macroseismic file was made (based on questionnaires sent out). MISSING means that the file has been lost. In May 2011, a check was made for missing ISO files. 44 files were regenerated from the original questionnaires. Original intensities of 2 and  2.5 were changed to 3 and 3.5 to 4.

| Date and time | ISO-file | Macroseismic | Comments |
| --- | --- | --- | --- |
|  |  | questionnaires |  |
| 20.01.1975 10.47 |  |  |  |
| 18.03.1975 13.15 |  |  |  |
| 03.04.1975 06.39 | ok | ok | map ok, 63 positive observations |
| 19.04.1975 11.48 | ok | ok | 6 positive observations |
| 26.05.1975 00.51 | ok | ok | 3 positive observations |
| 28.06.1975 10.42 | ok | ok | 4 positive observations |
| 03.10.1975 18.01 |  |  |  |
| 22.12.1975 02.53 | MISSING | ok | 1 positive observation |
| 02.01.1976 07.35 | Ok | ok | map ok |
| 18.01.1976 04.46 | Ok | ok | 4 positive observ. Svalbard |
| 02.03.1976 15.30 | MISSING | ok | 4 positive observations |
| 23.03.1976 04.44 | ok | ok | map ok, 29 positive observations |
| 29.03.1976 14.14 | ok | ok | map ok, 7 positive observations |
| 23.06.1976 15.59 |  |  |  |
| 18.08.1976 20.45 | Ok | ok | map ok |
| 09.10.1976 09.24 | Ok | ok | map ok |
| 18.11.1976 16.09 |  |  |  |
| 06.04.1977 19.31 | Ok | ok | map ok |
| 30.04.1977 23.32 | Ok | ok | map ok |
| 03.09.1977 12.19 | Ok | ok | map ok |
| 16.11.1977 23.44 | Ok | ok | map ok |
| 08.12.1977 09.05 |  |  |  |
| 11.12.1977 21.46 | Ok | ok | map ok |
| 08.01.1978 15.13 |  |  | Svalbard |
| 09.01.1978 02.47 | Ok | ok | map ok |
| 20.03.1978 03.59 | ok | ok |  |
|  |  |  | 23 positive observations from UIB |
| 25.04.1978 01.02 | ok | ok | map ok, 9 positive observations |
| 29.04.1978 21.11 | ok | ok | map ok, 39 positive observations |
| 22.05.1978 09.14 | ok | ok | map ok, 3 positive observations |
| 11.06.1978 09.15 |  |  |  |
| 19.08.1978 15.13 | Ok | ok | map ok |
| 12.11.1978 22.21 | Ok | ok | map ok |
| 28.12.1978 02.16 |  |  |  |
| 09.01.1979 20.13 |  |  |  |
| 28.03.1979 19.45 |  |  | map ok, only negative observations |
| 14.06.1979 21.24 | Ok | ok | map ok |
| 28.06.1979 04.25 | ok | ok | map ok, 4 positive observations |
| 22.07.1979 18.10 | ok | ok | map ok, 8 positive observations |
| 23.09.1979 13.52 |  |  |  |
| 09.10.1979 02.55 |  |  |  |
| 19.11.1979 00.33 |  |  |  |
| 19.11.1979 23.15 | ok | ok | map ok, 13 positive observations |
| 20.11.1979 17.36 |  |  | Jan Mayen |
| 06.01.1980 20.05 |  |  | Jan Mayen |
| 10.01.1980 16.03 |  |  |  |
| 14.03.1980 04.56 | Ok | ok | map ok |
| 08.06.1980 07.58 | Ok | ok | map ok |
| 18.11.1980 15.42 | Ok | ok | map ok |
| 08.12.1980 07.35 | Ok | ok | map ok |
| 01.03.1981 05.08 | Ok | ok | map ok , 1 positive observation |
| 09.03.1981 04.46 |  |  |  |
| 18.03.1981 16.29 |  |  |  |
| 03.09.1981 18.39 | Ok | ok | map ok |
| 06.09.1981 04.12 | Ok | ok | map ok, only negative observations |
| 04.01.1982 18.22 | Ok | ok | map ok |
| 07.04.1982 18.08 | Ok | ok | map ok |
| 19.04.1982 09.49 | Ok | ok | map ok |
| 20.04.1982 13.19 | ok | ok | map ok, 6 positive observations |
| 29.07.1982 00.17 | ok | ok | map ok, 9 positive observations |
| 06.08.1982 07.41 | Ok | ok | map ok |
| 10.10.1982 18.09 | Ok | ok | 1 positive observation |
| 15.10.1982 17.20 |  |  |  |
| 18.10.1982 02.58 |  |  |  |
| 15.12.1982 06.44 | ok | ok | map ok |
| 08.03.1983 18.43 | ok | ok | map ok |
| 12.05.1983 04.15 | ok | ok | map ok |
| 14.08.1983 23.51 | ok | ok | map ok |
| 18.08.1983 06.53 |  |  |  |
| 29.10.1983 17.51 | ok | ok | map ok, 10 positive observations |
| 15.11.1983 20.56 |  |  |  |
| 16.01.1984 21.23 | ok | ok | map ok |
| 11.03.1984 03.10 |  |  |  |
| 11.03.1984 03.36 |  |  |  |
| 08.05.1984 01.32 |  |  |  |
| 08.05.1984 01.33 |  |  |  |
| 27.07.1984 22.41 | ok | ok | map ok, 33 positive observations |
| 07.11.1984 11.12 | ok | ok | map ok |
| 25.12.1984 20.04 | ok | ok | map ok, 9 positive observations |
| 30.12.1984 23.36 | ok | ok | map ok, 44 positive observations |
| 06.01.1985 22.19 | ok | ok | map ok |
| 13.03.1985 21.07 | ok | ok | map ok |
| 27.11.1985 04.53 | ok | ok | map ok, 13 positive observations |
| 30.11.1985 19.05 | ok | ok | map ok, 13 positive observations |
| 05.02.1986 17.53 | ok | ok | map ok |
| 08.08.1986 17.06 | ok | ok | map ok |
| 26.10.1986 11.34 | ok | ok | map ok, 20 positive observations |
| 16.12.1986 01.30 | ok | ok | map ok |
| 04.02.1987 12.02 | ok | ok | map ok |
| 13.04.1987 18.57 | ok | ok | map ok , 1 positive observation |
| 25.05.1987 02.35 | ok | ok | map ok, 13 positive observations |
| 25.07.1987 05.30 | ok | ok | map ok, 12 positive observations |
| 19.10.1987 05.34 | ok | ok | map ok, 25 positive observations |
| 31.10.1987 10.09 | ok | ok | map ok, 38 positive observations |
| 01.11.1987 20.39 | ok | ok | map ok |
| 26.12.1987 16.57 | ok | ok | map ok, 3 positive obsvations |
| 31.01.1988 18.51 | MISSING | ok | map ok, 11 positive observations |
| 17.03.1988 18.58 | ok | ok | map ok |
| 08.08.1988 19.59 | ok | ok | map ok |
| 20.10.1988 21.43 | MISSING |  | map ok, 97 positive observations |
| 27.10.1988 14.22 | MISSING | ok | map ok , 1 positive observation |
| 13.12.1988 04.01 |  |  | Jan Mayen |
| 20.01.1989 09.33 | ok | ok | map ok, 29 positive observations |
| 23.01.1989 14.06 | ok | ok | map ok, 195 positive observations |
| 29.01.1989 16.38 | ok | ok | map ok, 131 positive observations |
| 14.02.1989 20.44 | ok | ok | 2 positive observations, map ok |
| 01.12.1989 16.25 | ok | ok | map ok |
| 19.01.1990 23.41 | ok | ok | map ok , 16 positive obs. |
| 25.02.1990 10.00 | ok | ok | map ok, 17 positive observations |
| 29.03.1990 04.11 | ok | ok | map ok , 9 positive obs. |
| 23.09.1990 00.35 | ok | ok | map ok, 17 positive observations |
| 30.10.1990 23.08 |  |  |  |
| 20.01.1991 19.27 | ok | ok | map ok |
| 23.01.1991 15.45 | ok | ok | map ok |
| 23.08.1991 04.07 | ok | ok | map ok, 2 positive observations, Sleipner accident |
| 28.10.1991 13.14 | ok | ok | map ok |
| 01.01.1992 08.03 | ok | ok | map ok |
| 19.02.1992 06.39 | ok | ok | map ok |
| 11.03.1992 22.05 | ok | ok | map ok |
| 11.04.1992 15.05 | ok | ok | map ok |
| 14.04.1992 13.10 | ok | ok | map ok |
| 18.05.1992 13.47 | ok | ok | map ok, 1 positive observation |
| 22.05.1992 08.40 | ok |  | map ok, 1 positive observation |
| 24.05.1992 14.48 | ok | ok | map ok |
| 28.06.1992 12.23 | ok | ok | map ok |
| 28.06.1992 20.56 | ok | ok | map ok |
| 03.08.1992 11.16 | ok | ok | map ok |
| 06.08.1992 07.32 | ok | ok | map ok |
| 09.11.1992 17.52 | ok | ok | map ok |
| 07.02.1993 22.17 | ok | ok | map ok |
| 06.03.1993 01.26 | ok | ok | map ok |
| 27.11.1993 18.57 | ok | ok | map ok |
| 27.11.1993 23.11 | ok | ok | map ok, 1 positive observation |
| 21.01.1994 01.26 |  |  |  |
| 26.01.1994 17.27 | ok | ok | map ok |
| 22.02.1994 05.55 | ok | ok | map ok |
| 27.07.1994 09.42 |  |  | 1 positive observation |
| 09.09.1994 01.02 |  |  |  |
| 22.09.1994 05.25 | ok | ok | map ok |
| 08.10.1994 21.02 |  |  |  |
| 19.11.1994 09.07 | ok | ok | map ok |
| 02.02.1995 21.26 |  |  |  |
| 06.02.1995 17.00 | ok | ok | map ok |
| 14.03.1995 15.29 |  |  | 1 positive observation |
| 17.03.1995 23.43 |  |  | 1 positive observation |
| 29.03.1995 22.26 |  |  | 1 positive observation |
| 29.03.1995 22.37 |  |  | 1 positive observation |
| 10.05.1995 23.42 | ok | ok | map ok, 3 positive observations |
| 21.05.1995 21.52 | ok | ok | 2 positive observations |
| 20.06.1995 21.22 | ok | ok | map ok |
| 03.07.1995 12.49 |  |  | 1 positive observation |
| 14.10.1995 04.32 |  |  | 2 positive observations |
| 23.10.1995 01.08 |  |  | phonecalls to Norsar |
| 13.11.1995 01.22 | ok | ok | map ok |
| 13.11.1995 08.09 | ok | ok | map ok |
| 20.11.1995 04.01 | MISSING | ok | 2 positive observations |
| 21.01.1996 02.16 | ok | ok | map ok |
| 17.03.1996 03.28 | ok | ok | map ok |
| 16.04.1996 10.27 | ok | ok | map ok |
| 31.10.1996 12.52 |  |  |  |
| 31.10.1996 12.57 | ok | ok | map ok |
| 11.11.1996 11.11 |  |  |  |
| 10.12.1996 10.42 | ok | ok | map ok, 1 positive observation |
| 16.12.1996 04.09 |  |  |  |
| 31.12.1996 16.11 |  |  |  |
| 28.04.1997 03.23 |  |  | 1 positive observation |
| 12.05.1997 22.45 | MISSING | ok | 2 positive observations |
| 13.05.1997 22.07 |  |  | Trollfeltet |
| 30.06.1997 11.25 |  |  |  |
| 12.08.1997 08.14 | ok | ok | map ok |
| 15.09.1997 15.58 | ok | ok | map ok |
| 13.12.1997 07.02 |  |  | Jan Mayen |
| 20.12.1997 21.40 |  |  |  |
| 01.01.1998 21.00 | ok | ok | map ok |
| 09.03.1998 14.19 | ok | ok | map ok |
| 13.10.1998 22.21 |  |  |  |
| 26.10.1998 23.06 |  |  |  |
| 28.11.1998 09.08 | ok | ok | map ok |
| 07.01.1999 14.04 |  |  |  |
| 17.01.1999 20.13 |  |  |  |
| 30.03.1999 09.21 |  |  |  |
| 10.04.1999 15.04 | ok | ok | map ok |
| 29.05.1999 00.31 | ok | ok | map ok |
| 22.08.1999 02.08 |  |  |  |
| 23.08.1999 14.11 |  |  |  |
| 13.10.1999 22.37 |  |  |  |
| 17.10.1999 12.07 | ok | ok | map ok |
| 01.02.2000 12.42 |  |  |  |
| 10.02.2000 06.17 | ok | ok | map ok |
| 13.03.2000 05.20 |  |  |  |
| 07.04.2000 03.38 |  |  |  |
| 27.04.2000 21.44 |  |  |  |
| 19.05.2000 16.59 |  |  | Jan Mayen |
| 21.05.2000 19.58 |  |  | Jan Mayen |
| 21.05.2000 23.42 |  |  | Jan Mayen |
| 22.05.2000 11.57 |  |  | Jan Mayen |
| 24.05.2000 01.10 |  |  | Jan Mayen |
| 25.05.2000 04.09 |  |  | Jan Mayen |
| 01.06.2000 17.08 | ok | ok | map ok |
| 05.07.2000 17.58 |  |  |  |
| 19.07.2000 17.28 |  |  | Jan Mayen |
| 12.08.2000 14.27 | ok | ok | map ok, Stord |
| 15.08.2000 05.00 |  |  |  |
| 01.09.2000 11.48 |  |  |  |
| 19.10.2000 10.27 | ok | ok | map ok |
| 26.11.2000 00.17 |  |  |  |
| 29.11.2000 02.22 |  | ok |  |
| 29.11.2000 09.59 | ok | ok | map ok |
| 29.11.2000 22.07 | ok | ok | map ok,Sweden |
| 08.12.2000 00.48 | ok | ok | map ok |
| 23.01.2001 10.04 |  |  |  |
| 19.02.2001 12.45 | ok | ok | map ok |
| 30.03.2001 14.52 |  |  |  |
| 31.03.2001 11.30 |  |  |  |
| 16.04.2001 03.28 |  |  |  |
| 07.05.2001 09.43 |  |  | Ekofisk |
| 19.07.2001 12.01 |  |  | Jan Mayen |
| 22.07.2001 02.40 |  |  |  |
| 07.06.2002 22.18 |  |  |  |
| 09.06.2002 06.44 |  |  |  |
| 27.06.2002 11.22 |  |  |  |
| 16.11.2002 12.27 |  |  |  |
| 21.11.2002 07.03 |  |  |  |
| 10.05.2003 21.17 |  |  | Rockfall Trollveggen |
| 27.05.2003 08.53 |  |  | Military exercise |
| 19.06.2003 12.59 |  |  | Jan Mayen |
| 28.06.2003 02.12 |  |  | Jan Mayen |
| 04.07.2003 07.16 |  |  | Hopen |
| 21.12.2003 00.25 |  |  | Masi, Finnmark |
| 21.12.2003 19.48 |  |  | Jan Mayen |
| 12.01.2004 07.11 |  |  |  |
| 15.02.2004 08.08 |  |  |  |
| 27.02.2004 18.11 |  |  |  |
| 07.04.2004 08.53 | ok | ok | map ok |
| 14.04.2004 23.07 |  |  | Jan Mayen |
| 02.05.2004 19.24 |  |  |  |
| 11.05.2004 00.50 |  |  |  |
| 29.06.2004 01.25 | ok | ok | map ok |
| 12.08.2004 16.36 |  |  |  |
| 15.08.2004 20.39 |  |  |  |
| 15.08.2004 20.48 |  |  |  |
| 20.08.2004 07.58 |  |  |  |
| 03.09.2004 09.31 | ok | ok | map ok |
| 21.09.2004 13.32 |  |  | Baltic Sea |
| 28.09.2004 11.53 |  |  |  |
| 01.11.2004 22.26 | ok | ok | map ok |
| 29.12.2004 06.34 |  |  |  |
| 04.02.2005 21.16 |  |  |  |
| 24.02.2005 08.39 |  |  | Military exercise, Battle Griffin |
| 01.05.2005 07.27 |  |  |  |
| 24.06.2005 04.25 |  |  |  |
| 25.07.2005 16.02 |  |  | Jan Mayen |
| 28.07.2005 20.29 | ok | ok | map ok |
| 13.10.2005 14.08 |  |  |  |
| 27.11.2005 23.16 | ok | ok | map ok |
| 15.12.2005 16.47 |  |  |  |
| 31.12.2005 02.48 |  |  |  |
| 11.05.2006 14.30 |  |  |  |
| 21.05.2006 18.13 |  |  |  |
| 03.07.2006 04.42 |  |  |  |
| 19.07.2006 20.42 |  |  |  |
| 28.10.2006 23.48 |  |  |  |
| 06.11.2006 23.32 |  |  |  |