Modifications in SEISAD18 boards

The current version of the PCB layouts for the SEISAD18 boards needs some modifications to be made, due to some bugs in the design. These are four, all of them regarding the power supply and ground lines in the SEISAD18 board. The PLL board does not need any modification.

The bugs can be fixed making the following connections in the SEISAD18 board:

- Pin 24 of integrated circuit IC5 (AD7710) to pin 6 of the same IC5.

- Pin 24 of integrated circuit IC4 (AD7710) to pin 6 of the same IC4.

- Pin 23 of IC3 (AD7710) to pin 14 of IC6 (PIC16F84).

- Pin 12 of IC3 (AD7710) to pin 1 of IC1 (MAX638).