SEISMIC HANDLER SHM

Beginner's manual

Seismic handler SHM (Motif version) is a program for routine analysis of

seismic data from an array or a network of stations. The data format

required is mini-seed, yet the program has capabilities of reading in

GSE.

The program has been written by Klaus Stammler of the Seismological Observatory

Graefenberg, Erlangen, Germany (em: klaus@szgrf.uni-erlangen.de)

This manual is not intended to be complete, as: 1) the author of this

manual is rather an SHM beginner, not an expert and 2) the author of

this manual uses SHM for a small network (4 stations SUW,WAR,OJC,KSP in

Poland), without accessing array and large network data. Therefore,

little if anything can be said here about array and location functions

of SHM. This manual is intended as an introduction to SHM.

1. Setup of SHM

================================

a. Environment variables.

SHM uses a number of environment variables to move around a directory

tree on a file system. The variables should be set in such a way as

to avoid any symbolic links. SHM usually does not recognize such links

and problems may result. The variables are:

PWD - current directory.

SEED_INPUTS - directory of calibration files

SEED_PROG - directory of seed I/O utilities

SFD - path to where are the data files.

SH_COMMAND - directory of command procedures of SHM

SH_COMMAND2 - the /bmp subdirectory of the above

SH_ERRORS - directory of error files

SH_EXTPROG - directory of external programs to be called from within SHM

SH_FILTER - directory of filter files

SH_GLOBALS - directory of network files and magnitude calculation files

SH_HELP - help files should be here

SH_INPUTS - directory of travel time tables, station lists

SH_LIB - directory of SH libraries

SH_ROOT - root directory of SH

SH_SCRATCH - directory to put scratch files

SH_SOURCE - directory of SH source files

SH_TEXTEDIT - text editor to use by SHM

SH_USERDIR - SH user directory

SH_UTIL - directory of SHM utilities

Other variables for advanced purposes:

SH_LOCSAT, SH_LOCSAT_EXEC, SH_PDE, SH_RANLIB

Some other variables are important during SHM installation:

SH_CCOPTIONS, SH_COMPILER, SH_LINKLIBS, SH_LINKOPTIONS

It is best to have these variables set in the user's .cshrc file.

b. Data preparation.

SHM reads mini-seed files, with extensions BH? (broadband 20Hz), LH?

(long period 1Hz) and HH (high frequency 80Hz). The files should be

gathered in some directory. SHM needs a file  sfdlist.sfd  which

determines what time segments are covered by each data file. This

file must be created in the directory by:

$SEED_PROG/sfdlist.csh '*.?H?' $PWD

or (if there happen to be too many files):

$SEED_PROG/sfdlist_l.csh '*.?H?' $PWD

Attention: when there are symbolic links on the data directory tree,

sometimes it may be necessary to manually setenv PWD to the current

full absolute directory path before running sfdlist.

Attention: once sfdlist is run, the SHM does not see any changes made

to the directory (as of its contents). If a data file is added after

the sfdlist command, SHM will not load this data. There is a solution

to this problem: without exiting SHM, it is possible to run sfdlist

from another terminal window - as long as it is run in the correct

directory.

Now we must setenv SFD to the path where are the data files (absolute

path, no symbolic links), e.g.:   setenv SFD /data/sdat/1996/01

We are now ready to start the SHM from a cmdtool or xterm window.

Attention: if there happen to show many, many messages on the screen,

there is probably something wrong with the SHM configuration and

you should consult an expert of SHM.

c. Starting SHM

To start SHM - assuming the env. variables are set up correctly and

the sfdlist.sfd created - just type in the name of the program in any

cmdtool or xterm window. The name of the program may be "shm", "SHM",

or "shm_world", depending on the version. (To clear out: shm_world is

a distribution version, shm is a test version, and SHM is usually the

alias for shm_world).

2. The SHM screen

=================

SHM opens two windows and two boxes on startup. The main, largest window

is the TRACES window, at bottom right. Above it opens the MAGNIFIER window.

The two boxes at left are the DIALOG box containing the phase information,

and the PARAMETER box, containing the location information.

The windows on startup are empty, only the TRACES window has a menu:

File Work Window Array Locate Param Amplitude TraceList Save Specials.

Below this menu, if signals are loaded, SHM displays the time of the

position of the mouse, signal length and last command. If it doesn't,

there might be something wrong with the startup configuration, so consult

a specialist.

The SHM is mostly mouse-oriented. The left mouse button is used mainly

for phase picking and to pick items from the boxes and menus. The right

button serves to mark a trace fragment to be magnified in the MAGNIFIER

window. Seldom needed is the keyboard - mainly to write in non-standard

phases, extremely long time series length, write in external location

and origin time (if known), specifying an external filter or command.

3. Data input

=============

The main data format used by SHM is mini-seed.

>From the menu, choose "File" - press left mouse button and drag the mouse

onto "Read" and let go. A window opens, from which we choose by left mouse

button:

a) The stations to be read in.

b) The components to be read in.

c) Type of data to be read in (LH-1Hz, BH-20Hz, HH-80Hz)

d) Date & time of beginning of time series.

e) Length of the time series. When less than 60 minutes, move the mouse

over the little horizontal bar and drag it left or right. When over

60 minutes, click the left mouse button in the little window and in-

put the number of minutes from the keyboard.

Usually that is all, so we click "Read New".

Attention: if this is not our first read in SHM, the program will not

read in data unless we select "Work->Cancel parameters" before selecting

"Read".

Attention: when during working with data you decide to read a longer/shorter

fragment of data, or starting at a later/earlier time, do not cancel your

parameters, but make changes in the "Read" window and finish with "Read

Again".

Reading in GSE

--------------

The author of this manual has not had the chance to work with SHM on GSE data.

These data can be read in by "File -> Read GSE" or "File -> Read GSE2", de-

pending on whether the data are GSE or GSE2.

4. Scaling of traces, offset removal

====================================

The input traces may need scaling. The change of scaling is done by

selection of "Specials->Setup->normalization". Then we may choose:

c - constant for all traces

all(tot) - normalized to maximum amplitude on any trace within all traces

all(wdw) - normalized to maximum amplitude on any trace within the

traces currently on display

single(tot) - normalized to maximum amplitude on each trace within

its full length

single(wdw) - normalized to maximum amplitude on each trace within

the visible window.

After selection, click on "Hide". The scaling type is remembered until

it is changed or until Quit from SHM.

The traces may be furtherly scales by use of the little "zoom" bar at

bottom of the PARAMETER box. Caution must be taken, one unit of "zoom"

means scaling by the factor of two.

The offset may be easily removed by chhosing "TraceList->Demean".

Similarly, spikes may be removed by "TraceList->Despike", however, these

must be real, electronic spikes, not banging of something in the vicinity

of the seismometer.

5. Use of filters

=================

To use filters, choose "Work->Filters". This opens a window with different

types of filters to use. Click on a little diamond left to choose a filter.

Some filters (Butterworths) need also some parameters like low bound and

high bound and filter order, also to be selected by mouse by clicking on

the little triangles near the values.

Choose a value for autocut to eliminate the rigning of the filter at the

beginning of the signal.

Use the "filter name" box to input an external filter name from the

keyboard, if needed (usually do not do anything in this box).

Having selected everything, click on "OK'.

To go back to unfiltered signals, choose "Work->Filters" and then click

on "none" and "OK".

Attention: After the filtration, the contents of the MAGNIFIER window

remains unchanged until the first click of the right mouse button.

6. Magnifying a fragment of a trace

===================================

To magnify a trace fragment move the mouse onto the beginning of the

fragment, click the right button and drag the mouse right to the end

of the fragment. There, let go of the button. The marked fragment will

be shown in the MAGNIFIER window. Only one trace at a time may be

magnified.

a) To move the MAGNIFIER window on the trace, move the mouse into the

center of the marked box on the TRACES window, click the right button,

and drag.

b) To move the MAGNIFIER window from trace to trace, move the mouse onto

the new trace and click once on the right button.

c) To change the width of the MAGNIFIER window, move the mouse onto the

right limit of the marked box, click right button and drag left or

right. At the desired width, let go. Then you may furtherly need to

move the window, see p.6a.

d) To empty the MAGNIFIER window so the box marked on the TRACES window

disappears (desired e.g. before printout!), choose

"Window->Del_time_window".

7. Phase picking

================

Phases are picked by means of the left mouse button on the seismic traces.

This may be done on filtered or unfiltered signals, in TRACES as well as

in MAGNIFIER window. Of course, picks made in MAGNIFIER window are much

more accurate, so this is the place the picks should be made. Phases picked

in the MAGNIFIER window are at once displayed in the TRACES window and vice

versa.

After the click, the phase is marked with parameters as are currelty

displayed in the DIALOG box. Therefore, before you click, make sure these

parameters are correct. Select or check the:

a) Phase name (selected from one of two submenus after clicking on "local"

or "tele". If needed, you can enter your own phase name into the little

box which shows the current phase name. To do this, click on the box,

and then use the keyboard.

b) The direction of first onset (+,- or 0). Of course this is relevant to

P waves on vertical component only.

c) Onset type (emergent or impulsive). In case of impulsive P waves on the

vertical component you should always give a non-zero direction of first

onset.

d) Reliable - should be "y". Other choice does not make sense for networks,

only for arrays.

e) Event type. Mark the event as local, regional, teleseismic, nuclear or

blast.

f) "Quality" and "More" - leave unchanged.

At the bottom of the box there are three rectangles: Del, Fix and Hide.

Hide causes the DIALOG box to disappear. For the box to reappear, you

must mark a phase (with the current parameters), and then try to delete

it. Better do not use "Hide".

Del causes all the phases with parameters identical to the ones currently

set in the box, to be deleted. This may be sometimes troublesome; if

you have marked let's say, a PS phase on six traces and want to delete

it from trace no. 4, you have to delete the phase (it disappears from

all 6 traces!) and then pick it again on traces 1,2,3,5 and 6.

Fix - The purpose of this function is unknown.

SHM may help in phase identification, showing phase onsets calculated

from theoretical tables. See p.12 how to do this.

Attention: SHM does not accept two phases of identical name on the same

trace.

8. Period and amplitude determination

=====================================

As the amplitude determination must be done on SRO-LP (or WWSSN_LP or

WWSSN_SP) filtered signals, you must first invoke the appropriate filter.

Don't worry. Your phase picks will remain unchanged, and you will be

able to go back to your current display after you have measured the

periods and amplitudes.

a) Choose "Work->Filter" and select SRO-LP (see p.5).

b) Mark the desired phase onsets or - in case of L waves, put phase

marks just before the fragment on which you will take your measures.

(It is best to do this in the MAGNIFIER window).

c) For each trace, choose "Amplitude->Ampl&Period_auto_peak". Then

move the mouse onto the first maximum (or minimum) after the pick,

click left button, move the mouse onto the next minimum (or maximum)

and click again. The part of the signal will change color to show

the measurement took effect. Repeat this for all desired traces.

d) After measuring the amplitudes and periods you may return to your

original filtration (see p.5).

Attention: SHM checks if the measured parameters are correct; if not, it

will not save them and will give a warning about this only to the window

it was started from.

9. Results of SHM processing

============================

a) The results, as phase picks, measured amplitudes, locations, magnitudes,

may be shown on the screen by means of "Work->Show Parameters". This

invokes a texteditor, so we may, if we want, make changes to the report

file using "Edit". You may now click on "File" by right mouse button and

select:

aa) Load file. Load another text file into the text editor. This makes

little sense at this point.

ab) Save current file. Save the current file to $SHSCRATCH/shm_number.evt

This makes sense only if you have made changes to the file.

ac) Store as New File. Save the file under new name to directory $PWD.

ad) Include file. Merge a disk file with our current file.

ae) Empty document.

To exit texteditor, click right mouse button on the little triangle on

the left of the top bar of the editor window. The choose "Quit". If you

accidentally click on the left mouse button, you will iconify the text-

editor, you must then double-click left on the icon, and then quit it

by right mouse button.

I haven't found useful purposes for the "View" and "Find" options of the

texteditor.

b) Before you save your final parameters, it might be useful to input your

comment to the event, such as: "Salomon Islands M=7.1", or "many spikes

on the E component at WAR". To input a comment, choose "Work->Comment",

or press a "v" with your mouse within the TRACES window. This invokes

the texteditor, so you may input your comment. After you have written

it in, click the right mouse button on "File" and select "Save current

file". Then quit the texteditor by clicking on "File" and selecting

"Quit".

c) The final parameters are written down when you choose "Work->Final

Parameters" or by pressing "Ctrl-P" with your mouse within the TRACES

window. This will write the complete information about the event and

the phases picked to file shm_yymmddnnn.evt, where yy=year, mm=month

dd=day, nnn=shm_generated_event_number, to directory as declared in

shmsetup defpath_evtout statement in one of the startup files (see p. 14).

This also invokes the texteditor, so you must click on its "File" and

choose "Quit" to close it.

10. Exit from SHM

=================

To exit SHM for a moment it is best to iconify it (click left on the

triangle at the left side of the TRACES window top bar). To bring it back,

double-click left on the icon.

To totally quit SHM, choose first "Work->Cancel Parameters" and then "Quit".

11. Printouts

=============

Printouts are generated by pressing "Ctrl-H". Then you have to click left

mouse button in the TRACES or MAGNIFIER window (depending on the window of

which you desire the printout), to have it printed. The printout will

contain all the currently displayed phase picks, magnifier box, or other

stuff if present on the window. Of course, the default printer must be

configured on the machine.

There are two print modes, a fast mode and a slow quality mode. The quality

mode uses the sunview utility, so it does run only on machines with sun-

view installed and running. The quality mode also needs two other programs,

rasttopnm and pnmtops installed in the $SH_UTIL. The change of modes may be

done interactively by "Specials->Setup->Hardcopy" and choosing "fast" or

"quality". This may be also set up in one of the setup files (see p.14).

12. Theoretical phase calculation and display

=============================================

To have the SHM display the theoretical phase onsets, the little boxes

of source location and origin time in the PARAMETER box must be filled.

On an array or an extensive network, SHM may perform a location of its

own. In other case you have to supply the data manually, coming from

some other source of information (bulletin). You move your mouse onto

each little box, click left button, and input the data from the keyboard.

Before you actually write in the data, make sure there are no contents

left in the little box. If there is, delete it with the Backspace key.

On first write-in, SHM will ask for the source of your information; you

can input anything here (except leaving this space blank or supplying

spaces).

This way supply:

a. Latitude (negative if South)

b. Longitude (negative if West)

c. Depth in km

d. Date. The format of the date is: 23-jul-1996

e. Time. The format of the time is: 23:45:08.4

Now calculate the Azimuth and Distance. To do this, choose "Locate->

External Location".

Now calculate the theoretical phase onsets by "Locate->Theo(Tables)" or

by pressing "Ctrl-t" from the keyboard. A phase menu window opens up, from

which we choose the phases we want the SHM to mark. (You cannot mark the

phases shown in gray, as there are no tables available for these, if you

want to use them, first you must supply the theo tables for these phases

to SH_INPUTS.) SHM proposes the main phases to make the choice easier.

After choosing all the phases needed, click on O.K..

The calculation may take a moment, depending on the number and type of

phases chosen. The phases are then displayed on the screen. To get rid

of them, just press "Ctrl-d" (or you may choose "Locate->Del_Theo"). To

bring them back, press "Ctrl-t" (or "Locate->Theo(tables)") and click on

"O.K." (SHM remembers which phases have been chosen last time).

The theoretical phases may differ somewhat from the actual phases seen.

This is due to: location errors, origin time errors, discrepancies

between the travel time tables and actual travel times along a path

specific for a given event and observation point, and travel time inter-

polation when distance and source depth fall between two table entries.

Final phase determination should be performed manually, the theo_phase

utility is just for help.

13. Other options (not discussed above)

=======================================

a. File. There are also possibilities of reading in .GSE format files and

external filters.

b. Work->Rotate. Rotation of seismograms to components pertaining to P, SV

and SH waves. I guess you need to fill in the PARAMETER box to do this,

similarly as for the theoretical phases.

Work->Info_source. Specification of information source to be written into

the "Final Parameters".

Work->Parameter_Box. Clicking on this causes the PARAMETER box to disap-

pear (if dispalyed) or reappear (if hidden).

c. Window. These options double the functions of the right mouse button in

the TRACES window. The "Del_time_window" causes the MAGNIFIER window

to be cleared and the box marking the fragment to be magnified to dis-

appear.

d. Array. Here are the array options, which are not discussed in this

beginner's manual.

e. Locate. Except the "->External Location" "->Theo_phases" and "Del_theo"

discussed in p.12, other options need array or extensive network data,

or some external files not supplied with SHM.

f. Param. I have not discovered the use of these options.

g. Amplitude. Used to determine the amplitudes, periods and magnitudes.

The "Amplitude->Ampl&Period_auto_peak" is best to determine the ampl-

itudes and periods (see p.8), but there are other choices you may

experiment with.

h. Trace List.

-> Demean - discussed in p.4.

-> Delete. Delete the traces selected.

To select the traces, move the mouse onto the trace number left of

the trace, and click left button.

After "delete", if you want to bring the traces back again, you

must do "File->Read", then click on "Read_Again".

-> Despike - discussed in p.4.

-> Rm trend. Remove trend. Works similarly to "demean", only the

method is different.

-> Ref ml. I have not discovered the purpose.

-> Hide Sel. Hide the traces selected. Works as delete, except that

if you want to bring the traces back, just choose the option again.

-> Hide Unsel. Works as Hide Sel, except that hides the traces NOT sel-

ected.

-> Sort Dist/Sort Azim - Putting the traces in order according to

distance or azimuth vs. the source.

i. Save. I have not discovered the use of these options.

j. Specials. These options are rather seldom used by the SHM beginner. The

most important of these is the "Setup". Here can be found the:

-> Special commands. The instructions here depend on the contents of

some configuration files. You can for example interactively change

your seismic network here.

-> Del Horiz. Delete horizontal components. Works as "Trace_List->Delete",

except that it does not need trace selection. This command has a short-

cut "Ctrl-O" from the keyboard.

-> Screendump. A shortcut for this is "Ctrl-H", discussed in p.11.

-> User Cmd. Gateway to give a user command. Advanced option.

-> Command. Gateway to give an extra SH command. Advanced option.

-> Setup:

--> hardcopy fast / quality. Set type of printouts.

--> max cursor. I haven't discovered the use of this.

--> normalization - discussed in p.4.

--> trace order. Display the traces with trace no.1 at the top or at

the bottom. This can be also set in SHM_USER_STARTUP.SHC file

as "shmsetup top_down_order true" or "...false". The "false" is

the default.

--> filter type. Rather an advanced option.

--> default filter. If used, all the traces are filtered with this

filter after being read and before being displayed.

--> parameter list. I haven't discovered the use yet.

--> phase names. I haven't discovered the use yet.

14. Elements of SHM configuration

=================================

The SHM to work needs a certain directory structure on the hard disk on the

computer, governed by the environment variables explained in p.1. Below some

of the elements of the configuration will be discussed.

A. User configuration.

There are two configuration files:

SH_COMMAND/SHM_CMD_STARTUP.SHC - standard configuration file

SH_USERDIR/private/SHM_USER_STARTUP.SHC - user configuration file

Both may have similar contents, but if both specify the same parameter,

the value from the user config is taken. If the user config does not

contain some parameter, the value of it is taken from the standard con-

fig, if that does not specify it, default value is taken.

The two files are executed only on startup of SHM.

In the files, lines starting with "!" are comment lines and are ignored.

Some of the contents:

shmsetup prompt_analyst true - on startup, this prompts the analyst to

identify himself.

sdef tmp - define variable tmp

@CALC s &TMP = \path_to\evid\ DOSLASH

shmsetup defpath_evid "tmp - The two above lines define the path where

SHM keeps track of event numbers used.

@CALC s &TMP = \path_to\evtout\ DOSLASH

shmsetup defpath_evtout "tmp - The two above lines define the path where

Final Parameter files are to be written.

shmsetup refstation suw - set reference station to 'SUW' (must be in

the list!)

shmsetup default_source igf - set default information source to 'igf'

shmsetup dephault_phase_type tele_quake - after read, the event is

qualified as teleseismic, until this qualification is changed in

the DIALOG box (see p.7e).

shmsetup default_filter name - prefiltration of signals, see p.13j

(setup->default_filter)

shmsetup default_depth_type preset - after read, a constant value will

be put to "Depth" window in the PARAMETER box.

shmsetup default_depth 33.0 - the above value is to be (in this

case 33 km).

shmsetup top_down_order true - order of traces on display. If true, trace

1 is at the top, if false, it is at the bootm.

shmsetup double_click_time 300 - double-click time for the mouse

shmsetup color_mark 1.0,0.0,0.0 - color (rgb) of phase picks (here:red)

shmsetup color_theo 0.0,0.0,1.0 - color of theo_phases (here:blue)

shmsetup color_auto 1.0,0.0,1.0 - color of amp&period determ.(here:violet)

shmsetup color_crsr 0.0,0.0,1.0 - color of cursor (here:blue)

@SHMSETUP SCREENDUMP_PROC |"TMP|sd_xwd.csh| - set printout as fast.

@SHMSETUP SCREENDUMP_PROC |"TMP|sd_screendump.csh| - set printout as

quality. Attention: only one of the two above statements should

appear in a config file. If there will be two, the first will be

ignored.

shmsetup defpath_data sfd - path to data files is given by the variable

sfd.

shm_cmd_norm sw - normalization of traces after read (here: sw=all channels

separately, within the visible window).

SHM_CMD_READ_DIALOG STATLIST_something.STX

STATLIST_something.STX is the file containing the list of station

codes belonging to the default network. The file is in directory

SH_GLOBALS.

B. Directory SH_GLOBALS

a. STATLIST_something.STX files contain the lists of station codes, be-

longing to networks. If there are several networks possible, there

will be several such files.

b. refstation_list.stx - list of possible reference stations (maximum 5).

c. Other files are used do determine magnitudes, filters and display

colors.

C. Directory SH_FILTER

Contains different filter definitions.

D. Directory SH_INPUTS

a. *.TTT files are travel time tables for theoretical phase calculation.

b. filter_lookup.txt. Definition of how different stations are to be

filtered. If the station is not here, there may be problems with

having it filtered.

c. list_of_analysts.txt. List of ID's of analysts.

(valid if shmsetup prompt_analyst true)

d. STATINF.DAT - All possible station codes and locations throughout the

world.

e. STATLOC.DAT - Station codes and locations; this data is used in location

procedures.

f. Some other files of unknown purpose.

E. Directory SEED_INPUTS

Among other files, there are the calibration files, e.g.:

seedcalib_suw-bh-z , containing calibration data for station (SUW),

trace type (bh=broadband), component (z).

15. Setbacks. These are few, but they do exist - as in almost every

software program. Those I found are:

- Enigmatic error communication: Error CL-06, with no further explanation.

- The "Help" usually doesn't give any help, at least in the current version.

- No possibility to integrate/differentiate signals.

This short manual has been written by Pawel Wiejacz of the Institute of

Geophysics, Polish Academy Of Sciences, em: pwiejacz@seismol1.igf.edu.pl ,

based on own experience with SHM.