#### Mountainair Development Plan

This document contains information that is proprietary to Quanterra, Inc. Use of the information in this document other than an informational description of a Quanterra product in development is strictly prohibited. Photocopying or electronic transmission of this document with the prior written permission of Quanterra, Inc. is prohibited.

The Mountainair software is an implementation of a Q330 Data Processor (DP) system. The Mountainair program registers on a Q330 logical telemetry port, acquires incoming data packets on a network interface, and outputs received data into a comserv memory area. By packing the received waveform data into 512 byte data packets, and by supporting the comserv API, the Mountainair software allows existing, un-modified, comserv clients to acquire data from a Q330.

In addition, the Mountainair software provides functionality previously performed by the datalogger, such as event detection processing. The Mountainair software also supports a limited set of comserv-client to Q330 commands through the comserv API.

Mountainair will also provide a Data Subscription Service (DSS) server for each logical telemetry port by implementing a data subscription server on the DP system.

Development Planning:

This plan reflects current requirements and priorities for the Mountainair software development effort. This plan will be updated during the course of development, as requirements and priorities change.

Development Process:

In order to provide high quality software in a reasonable amount of time, the Mountainair software development will use a lightweight development process based on the “Agile”, or “XP” methodologies. Features of these methodologies we intend to use during development include: (a) design as simply as possible while meeting requirements, (b) build small releases insuring short time between releases, (c) test the software using a testing framework that can run an automated suite of unit tests, and (d) keep the automated test suite working throughout the development process to avoid broken builds.

Target OS Platform:

The initial target platform is Solaris 2.7 and later. Issues relating to development and porting to other systems such as Linux with be considered during development but will not be addressed until a Solaris release is available.

Development Languages and Tools:

In order to make the software source code as accessible and portable as possible, Mountainair will be written in either C or C++.

It is not yet determined whether the primary development tools will be gcc, and or Sun Forte development Tools.

In order to facilitate portability, usability, and maintainability, an effort will be make to keep the code “generic”. No special libraries or tools will be introduced except for operating system provided libraries such as networking or math libraries.

Development Approach:

Our intention is to get a simple but flexible program framework running early, and then to enhance this framework during development. The framework should be simple enough to work with while we investigate design ideas, and while we verify our ability to correctly communicate and interoperate with the Q330. The framework should be flexible enough to support the enhancements needed to meet the final requirements.

In this section we outline our current thinking on the order of development of various capabilities and features. This should provide a basic idea of how we expect the development to proceed.

The capabilities and features outlined here are based on an initial review of the capabilities required by the Mountainair software, particularly in comparison to the Socorro and Willard systems, as well as on discussions with Q330 developers and users.

- Setup

- Establish a development environment with compiler, debugger, source code versioning, and an automated testing framework.

- Write utilities for handling C and Pascal differences, byte-swapping routines, CRC, and MD5 calculations.

- Develop data structures and utility programs for assembling and disassembling QDP packets.

- Write a Mountainair main program that reads a comserv initialization file, creates a comserv memory area, and opens full duplex UDP communications using an IP and Port number read from the configuration file. This will be the framework that the remaining Mountainair functionality is developed within.

- Develop status and logging capability for Mountainair. This will allow the program to report errors, diagnostic, and verbosity in a standard way.

  - Registration, Initial Status, Tokens, and Command Generation

- Develop the ability for Mountainair to register with a Q330 based on information in the Mountainair configuration file information including IP, and port.

- Develop the ability to format a status request, to send it to the Q330, and to receive and process the response.

- Generate token requests.

- Receive and process received tokens.

- Generate periodic status requests in Mountainair and send them to the Q330 based on a period specified in the Mountainair configuration file.

- Transmit a calibration command from Mountainair to Q330 based on a dp-da client command request, and verify Mountainair receives a response.

  - Initial Data Packet Handling

- Receive data packets, or data packet sequences.

- Decompress the data in the packets.

- Assembly 512 byte, Steim 2 compressed, Mini-seed packets and output to comserv data memory area.

- Verify and validate that a comserv client (datalog) can log the Mountainair data to disk.

  - Comprehensive Data Packet Handling

- Implement handling of all supported data rates with appropriate decimation and filtering (if required).

  - Reliable Delivery over UDP, and Data Gap Handling

- Implement and validate that the Q330 to Mountainair communication protocol is properly implemented including the “partial window acknowledgment”.

- Demonstrate proper handling of data packets when there missing packets due to communication failures.

  - Blocking Client Support

- Implement Mountainair support for blocking client behavior, to the extent is possible with the Q330 protocol.

  - Detection Processing

- Implement detection processing in Mountainair including filtering

- Create event detection packets and output to the Mountainair comserv memory area.

  - Establish DSS Interface to Waveform Data

- Establish how the DSS will access waveform data for DSS processing. At this point, establish how the DSS server will access the real-time data from the Q330. Full DSS capability will be developed later.

  - Timing and Status Logs

- Using the periodic status requests from Mountainair to Q330, and the status responses from the Q330, correctly interpret the received status information from the A330 and map it to MSEED status messages.

- Create valid MSEED timing and status packets from Q330 information.

- Verify logging of status and information timing information by comserv clients.

  - Command processing

- Fully implement the dp-da command handling including proper generation of Mountainair to Q330 message set. Verify that Mountainair responds with a no-op response to unsupported comserv commands.

- Implement Mountainair handling of Q330 responses to comserv-client commands. Map received Q330 messages into appropriate MSEED data packets and output to comserv memory area.

  - Portability, Network Flexibility

- Implement SLIP capability into the Mountainair program.

- Demonstrate a variety of networking approaches including UDP, SLIP, and multicast.

- Verify portability to a Linux little-endian platform.

- Verify that Mountainair software compiles using an alternative compiler.

  - DSS Processing

- Implementation of Data Subscription Server.

Documentation:

At this time the following documents are anticipated:

- Users Guide - How to build, install, configure, and run the software.

- Knowledge base - Things learned during implementation.

- Example Configuration Files

- README - Contents of distribution package.

End User Software Distribution:

The Mountainair software will be packaged for distribution in tar files. Two different collections of software and source code will be available.

- Complete development package, including the testing framework, the test codes, and all file needed to compile the software with the exception of operating system libraries.

- Developer’s package without the test framework, and test routines. Allows a user to compile the software on their system.

Distribution During Development:

Source code will be package on a regular basis and posted to an external ftp, or web server, for public review and comment. Documentation is expected to be in PDF format or possibly HTML and will also be posted as developed.