#### Mountainair Design Document

The Mountainair software is an implementation of a Q330 Data Processor (DP) in the “fixed system” category, as described in the Q330 DP Writers guide. The Mountainair software will register with a Q330, and process data packets, outputting data into a comserv memory area. The Mountainair software will allow existing, un-modified, comserv clients to acquire data from a Q330. In addition, Mountainair should provide support for DSS capabilities for Q330.

There are a number of software architectures under consideration for the Mountainair software.

These design approaches identify the number of processes and the IPC methods involved when recording data from a Q330.

The approaches described below are intended to prompt discussions and to help me understand how the Q330 designers envision the system working.

The Q330 can support up to 4 logical telemetry ports with different data sets from the same instrument. One instance of the Mountainair software will run for each logical telemetry port. At this point, a “Mountainair instance” may be one, or more, processes. The process count is the number of processes to populate a comserv memory area, and to provide a DSS server for the station.

In these diagrams, Circles are processes, enclosed rectangles are shared memory areas. Larger rectangles enclose processes which must run on the same computer. Arrows indicate IP-based communication between processes. Shared memory areas support two way communication between processes.

Approach A :

This approach designs two stand alone programs, one a comserv-proxy, one a mountainair dss server. Two logical ports are required if both comserv and dss are to be used for this logical port.

Comserv Proxy Process

Comserv

Shared

Memory

Q330

Comm

Process

Data Subscription Server

In Approach A, a fairly small program (called the comserv-proxy) which communicates with the Q330 on a virtual data port, repacketizes the Q330 data, and populates a comserv memory area with data. Comserv client to Q330 data is also supported.  The comserv proxy behaves very much like comserv, however it re-packetizes data as it receives it.

If the user wants to run DSS on this Q330, another program, Mountainair-Data Subscription Server (ma-dss) is started. This program communicates on a different virtual port with the Q330.

Advantages.

Comserv-proxy looks very much like a comserv. Two process per station to run comserv and dss. Simpler architecture making faster development. Lightweight approach may be appropriate for networks with many Q330s. No new shared memory area’s on host system.

Disadvantage:

When both comserv-clients and dss-clients are run on the same station, this approach requires two data streams, and two virtual ports, out of the Q330.

Approach B:

In  approach B, an on boarding program is designed which

Mshear packetizer and comm. proxy

Comserv Process

Comserv

Shared

Memory

On-boarding Process

Fixed Size Data packets

Q330

Comm

Process

Data Subscription Server

In approach B, an on-boarding processes runs for  each Q330 station. The on-boarding process receives the packets from the Q330, and puts them into a shared memory process. This shared memory is accessible by both a packetizing process and a dss. The packetizer is written to simulate mshear comm with comserv. By imitating mshear, it can send data to an unmodified comserv over a socket. This allows the onboarding to be done on a different machine than the comserv. A data subscription server can run using the onboarding shared memory.

Advantages:

Only one data stream and virtual port from a Q330 to support both  comserv and dss. Uses comserv unmodified.  Flexible process placement with network comm between mshear proxy and comserv.

Disadvantages:

Four processes for data acquisition per station. Two shared memory areas per station. Software involves two networking protocols. Properly handling all the conditions for each protocol complicates the software.

Approach C:

Packetizer and comserv proxy

Q330

Comm

Process

On-boarding Process

Data Subscription Server

Fixed Size Data packets

Comserv

Shared

Memory

Approach C is similar to Approach B. However in this design, the a comserv-proxy process is written which reads the on-boarding programs shared memory area, and outputs to a comserv shared memory area. Now, the comserv-proxy and the on-boarding program must run on the same machine.

Advantages:

Three process per “station”. Do not have to implement the mshear-comserv network protocol.

Disadvantages:

Requires a developing a comserv-proxy. Comserv-proxy must run on same host as on-boarding system. Two shared memory areas per station.

Q330

Comm

Process

Approach D:

Comserv

Shared

Memory

Packetizer and Comserv Proxy

Multi-threaded Data Server

Data Subscription Server

Approach D, involves a multithreaded server which manages incoming data and client connections. Because connections to this data server are through network connections, not shared memory is used. This is similar to the ORB approach.

Advantages:

Three processes per station. One shared memory area per station, the comserv shared memory.

Disadvantages:

The multithreaded server is a complicated program. Coupling of data reception and data distribution. Excessive client data requests can impact data reception. Must develop a protocol between the server and the comserv-proxy and ma-dss clients.

Approach E :

Q330

Comm

Process

Data Subscription Client

Comserv Proxy and Data Subscription

Server

Comserv

Shared

Memory

In this approach, the comserv proxy process acts as both a client, and a server. The comserv proxy acts as a client to the Q330, and as a server to the DSS server.

The complication here seems to constructing the comserv-proxy to work well as both a client and a server. Each network interface seems to have its own protocol, and must perform connection open and closes, and operations when connections are open.

Advantage: Resources required are a two processes and a single shared memory area.

Disadvantage: The complication of the comserv-proxy is increased, specifically relating to the two network connections. In addition, the comserv-proxy is now monolithic, containing both the comserv and DSS functionality.

Q330

Comm

Process

Approach F:

Data Subscription

Server

Comserv Proxy

Comserv

Shared

Memory

In this approach, the Data Subscription Server is implemented as a comserv client. The advantage is that the comserv-proxy and the DSS are implemented separately. In order to support the low latency, data packets will have to be put into the comserv memory area.

Advantages:

Disadvantages: