GeoSys - Bergen University serial data protocol (GSBU_GBV data protocol)

Specifications:	1 second data protocol

1 channel or 3 channels

38400 baud, 8 data bits, 1 start bit, 1 stop bit, no handshaking

Total amount of data:

25 Hz, 1 channel:	138 bytes	transmission time  36 ms

25 Hz, 3 channels:	288 bytes	transmission time  75 ms

50 Hz, 1 channel:	213 bytes	transmission time  56 ms

50 Hz, 3 channels:	513 bytes	transmission time  136 ms

100 Hz, 1 channel:	363 bytes	transmission time  95 ms

100 Hz, 3 channels:	963 bytes	transmission time  251 ms

200 Hz, 1 channel:	663 bytes	transmission time  173 ms

200 Hz, 3 channels	1863 bytes	transmission time  485 ms

Description:

| Offset | Bytes | Data | Format |
| --- | --- | --- | --- |
| 0 | 2 | “GB” (synch-characters) | ASCII characters |
| 2 | 2 | Device ID | Binary: Station address |
| 4 | 2 | Firmware rev. | Binary: |
| 6 | 2 | Status information | Binary: 0x0001: Sensor V 0x0002: Sensor NS 0x0004: Sensor EW 0x0008: -- …. 0x8000: -- General: 0: not OK 1: OK |
| 8 | 32 | Device information | Binary: |
| 40 | 4 | Block num. | Binary: (Seconds of a day counter, from 0 (0:00:00) … 86399 (23:59:59)) |
| 44 | 2 | Bytes per packet (bpp) | Binary (Data bytes): 25 Hz, 1 channel - 75 bytes 25 Hz, 3 channels - 225 bytes 50 Hz, 1 channel - 150 bytes 50 Hz, 3 channels - 450 bytes 100 Hz, 1 channel -300 bytes 100 Hz, 3 channels - 900 bytes 200 Hz, 1 channel - 600 bytes 200 Hz, 3 channels - 1863 bytes |
| 46 | 1 | A/D converter indicator | Binary: Indicates A/D converter type, order of converted data (low - mid. - high or high - mid - low and if the data comes with binary offset or 2’s complement. 0x01: 16 Bits out of 24 Bits (1st: high, 2nd: low, 3rd: not used), offset binary . 0x02: 22 Bits out of 24 Bits (1st: high, 2nd: mid, 3rd: low (only 6 bits used from the low one, Bits 0x01 and 0x02 not used), 2’s complement. 0x03: 24 Bits out of 24 Bits (high, mid, low), 2’s complement. |
| 47 80; 40; 20; 10; 08; 04; 02; 01. active channels. -; -; -; -; -; E-W; N-S; V. | 1 | Active channel indicator (low) 80; 40; 20; 10; 08; 04; 02; 01. active channels. -; -; -; -; -; E-W; N-S; V. 80; 40; 20; 10; 08; 04; 02; 01. active channels. -; -; -; -; -; E-W; N-S; V. 80; 40; 20; 10; 08; 04; 02; 01. active channels. 8; 7; 6; 5; 4; E-W; N-S; V. | Binary: 0: Channel n not active 1: Channel n active Channel 1: V Channel 2: NS Channel 3: EW |
| 48 80; 40; 20; 10; 08; 04; 02; 01. active channels. 16; 15; 14; 13; 12; 11; 10; 9. | 1 | Active channel indicator (high) | Binary: 0: Channel n not active 1: Channel n active |
| 49 | 10 | Date and Time | 1 byte time-system indication (binary) (0x01: 1pps 0x02: GPS update 0x04: RTC General: 0: not valid 1: valid) 2 bytes (BCD): year 1 byte (BCD): month 1 byte (BCD): day 1 byte (BCD): hour 1 byte (BCD): min 1 byte (BCD): sec 2 bytes (BCD): msec |
| 59 | 1 | Sample 1 high V | Binary |
| 60 | 1 | Sample 1 mid V | Binary |
| 61 | 1 | Sample 1 low V | Binary |
| .... | 1 |  | Binary |
| .... | 1 |  | Binary |
| .... | 1 |  | Binary |
| ... | ... | ... | ... |
| bpp + 58 | 1 | .. | Binary |
| bpp + 59 | 1 | Checksum / CRC | Binary: 4 bytes of checksum or CRC 2 Bytes checksum implemented, initially 0, includes sum from byte 0 ... byte bpp + 58 Checksum high byte |
| bpp + 60 | 1 | Checksum / CRC | Checksum low byte |
| bpp + 61 | 1 | Checksum / CRC | not used |
| bpp + 62 | 1 | Checksum / CRC | not used |